IoT-Enabled Smart Monitoring System for MCB Boards

Dhaarani.S Nethra P Ranjitha S Nivetha R

Bachelor of Engineering, Department of Electronics and Communication

R.M.K College Of Engineering And Technology

Abstract — In the realm of electrical systems, the reliable functioning of Miniature Circuit Breaker (MCB) boards is paramount. This project introduces an innovative solution that leverages Internet of Things (IoT) microcontrollers, specifically Arduino Uno, Nano, and Node MCU, to continuously monitor and report the trip status, temperature, and humidity of an MCB board. The system establishes a network of communication between the Arduino Uno, Nano, and Node MCU, enabling seamless data exchange for real-time monitoring. Through serial communication, these microcontrollers collaborate to collect and transmit vital information regarding the MCB board's operational status. The data is then seamlessly displayed on an Adafruit webpage, providing users with a user-friendly interface to monitor the MCB board remotely.

To enhance data integrity and enable post-analysis, the system incorporates an SD card module. Changes in the MCB board's status are recorded and stored on the SD card, ensuring a comprehensive and historical record of the system's performance. This feature not only facilitates data storage but also enables users to analyze trends and identify patterns over time. The utilization of Arduino Uno, Nano, and Node MCU ensures a cost-effective and scalable solution. These microcontrollers are well-suited for IoT applications, offering a balance between computational power and energy efficiency. The modular nature of the system allows for easy integration and expansion, catering to the evolving needs of diverse MCB installations.

This project presents a sophisticated IoT-based monitoring system that addresses the crucial task of overseeing MCB boards. Through the collaboration of Arduino Uno, Nano, and Node MCU, coupled with SD card data logging, the system provides a holistic solution for real-time monitoring, remote accessibility, and historical analysis of MCB board performance. This innovation contributes to the advancement of smart electrical systems, ensuring enhanced reliability and efficiency in monitoring critical components.

I.INTRODUCTION

In the realm of electrical systems, ensuring the seamless                   operation of Miniature Circuit Breaker (MCB) boards is paramount for maintaining safety and efficiency. To address this, our project employs cutting-edge Internet of Things (IoT) technology, integrating Arduino Uno, Nano, and NodeMCU microcontrollers for real-time monitoring and data analysis.

The central objective of our project is to continuously monitor the trip status, temperature, and humidity of the MCB board. The use of Arduino Uno, Nano, and NodeMCU facilitates a cohesive system where these microcontrollers communicate via serial communication protocols. This allows for a robust and synchronized monitoring mechanism, ensuring that critical data is transmitted accurately and in real-time.

To enhance the accessibility and usability of the monitored data, we leverage the Adafruit platform to create a dynamic webpage. Users can effortlessly access the status of the MCB board through an intuitive interface on the Adafruit webpage. This not only ensures real-time monitoring but also provides a user-friendly experience for quick insights into the MCB board's health.

Additionally, our system incorporates the utilization of an SD card module, enabling the storage of changes and data logs. This feature empowers users to analyze historical data, offering valuable insights into trends and patterns. The SD card serves as a comprehensive repository, ensuring that data is not only monitored but also archived for future reference.

The implementation of MQTT (Message Queuing Telemetry Transport) and TCP (Transmission Control Protocol) protocols enhances the project's scalability and adaptability. The Publish-Subscribe method further streamlines communication between devices, creating a responsive and efficient monitoring system.

II. SYSTEM DESIGN

The MCB Board Monitoring System is designed to provide real-time monitoring of the MCB board status, temperature, and humidity using IoT microcontrollers such as Arduino Uno, Nano, and NodeMCU. The system utilizes serial communication between Arduino Uno and Nano, and NodeMCU, ensuring seamless data exchange.

HARDWARE COMPONENTS

1.Arduino Uno: Monitors MCB board trip status, temperature, and humidity.

2. Arduino Nano: Communicates with Arduino Uno through serial communication.

3.NodeMCU: Connects to the MQTT broker and communicates with Arduino Uno via serial communication.

4.SD Card Module: Enables storage of MCB board data changes for later analysis.

5.Sensors: Temperature and humidity sensors connected to Arduino Uno for environmental monitoring.

COMMUNICATION PROTOCOLS

1. Serial Communication: Arduino Uno communicates with Arduino Nano and NodeMCU using serial communication for efficient data transfer.

2. MQTT Protocol: NodeMCU uses MQTT for real-time data transmission to an MQTT broker.

3. TCP Protocol: Facilitates communication between Arduino Uno and NodeMCU for reliable data exchange.

DATA STORAGE AND ANALYSIS

The SD Card Module stores MCB board data changes locally, providing users with the option to analyze historical data. The stored data includes trip status, temperature, and humidity readings.

WEB INTERFACE

The system leverages Adafruit's platform for creating a user-friendly web interface. Real-time data is displayed on the Adafruit webpage, allowing users to monitor MCB board status remotely.

PUBLISH-SUBSCRIBE METHOD

MQTT's publish-subscribe method ensures efficient and scalable communication between the components. Arduino Uno publishes MCB board data, and NodeMCU subscribes to receive and transmit the data to the MQTT broker.

This comprehensive system design ensures seamless integration of Arduino Uno, Nano, and NodeMCU for MCB board monitoring. The combination of MQTT and TCP protocols, along with SD card storage, provides a robust and scalable solution for real-time monitoring and historical data analysis. The user-friendly Adafruit webpage interface enhances accessibility, making it an effective IoT-based MCB board monitoring system..

Fig. 1: Overall Block Diagram of System

WORKING PRINCIPLE

The IoT-based MCB Board Monitoring System is designed to provide real-time monitoring and data analysis for electrical distribution systems. The core components of this project include Arduino Uno, Arduino Nano, NodeMCU, SD card module, and Adafruit platform. The system is capable of continuously monitoring the trip status of the MCB (Miniature Circuit Breaker) board, as well as ambient temperature and humidity levels.

The Arduino Uno acts as the primary controller, overseeing the entire system. It establishes a serial communication link with both Arduino Nano and NodeMCU to gather data from different sections of the MCB board. The collected information is then transmitted to the Adafruit platform via MQTT (Message Queuing Telemetry Transport) protocol, enabling users to access real-time data through a dedicated webpage.

To enhance data integrity and provide an offline analysis option, an SD card module is integrated into the system. The Arduino Uno writes the monitored data onto the SD card, allowing users to retrieve and analyze historical information. This feature is particularly useful for identifying patterns, anomalies, and potential issues in the electrical distribution system.

The system employs the MQTT protocol for efficient communication between the Arduino Uno and NodeMCU. MQTT's publish-subscribe method facilitates seamless data transfer, ensuring that updates from the MCB board are promptly relayed to the monitoring system. Additionally, the system utilizes TCP (Transmission Control Protocol) to establish a reliable communication channel, enhancing the robustness of data transmission.

Overall, this IoT-based MCB Board Monitoring System offers a comprehensive solution for real-time monitoring, data storage, and analysis. By combining the strengths of Arduino microcontrollers, SD card storage, and MQTT/TCP protocols, the system provides a versatile and effective means of ensuring the safety and reliability of electrical distribution systems.

III. RESULT AND ANALYSIS

The project incorporates continuous monitoring and display of the MCB board's trip status, temperature, and humidity on an Adafruit webpage. The seamless serial communication between Arduino Uno, Nano, and Node MCU ensures real-time data exchange.

Furthermore, a crucial feature involves data storage using an SD card module, enabling users to analyze historical data conveniently. The project utilizes both MQTT and TCP protocols, demonstrating its versatility in communication methods. The publish-subscribe mechanism enhances communication efficiency. The successful integration of these technologies ensures robust monitoring, data storage, and analysis capabilities for enhanced electrical system management. This project marks a significant step towards smart and efficient electrical infrastructure monitoring.Top of Form

CONCLUSION

The successful implementation of this project underscores the efficient monitoring and management of MCB boards through IoT microcontrollers such as Arduino Uno, Nano, and Node MCU. The real-time tracking of trip status, temperature, and humidity, showcased on the Adafruit webpage, exemplifies the project's commitment to transparency and accessibility. The incorporation of serial communication between Arduino Uno, Nano, and Node MCU ensures seamless data exchange. The utilization of an SD card module facilitates data storage for later analysis, providing users with a comprehensive understanding of MCB board behavior. The integration of MQTT and TCP protocols enhances communication reliability and security. The adoption of the publish-subscribe method further streamlines information flow. Altogether, this project not only elevates MCB board monitoring but also exemplifies a robust and scalable IoT solution, paving the way for enhanced electrical system management.

REFERENCES

[1] Brent L. Carper, Principal Engineer, “The X, Y and Z of Circuit Breaker Control”, 34th Annual Hands-On Relay School, March 2017.

[2] Jason M. Byerly, Carey Schneider, Robert Schloss, Isaac West “Real-Time Circuit Breaker Health Diagnostics,” 43rd Annual Western Protective Relay Conference, October 2016. [3] M. Kezunovic, G. Latisko, N. Ved, “Real-Time Monitoring and Assessment of Circuit Breaker Operations for Diagnostics and Control Applications”, Texas A&M University, College Station, TX 77843- 3128.

[4] Payman Dehghanian, Tomo Popovic, Mladen Kezunovic, “Real Time Circuit Breaker Operational Health Assessment via Condition Monitoring Data”, Department of Electrical and Computer Engineering, Texas A&M University College Station, Texas, 77843, USA.